Software Design Document

UART Driver Module

Lab 05 – Microcontrollers and Embedded Systems

| Module | UART Driver |
| --- | --- |
| Version | 1.0.0 |
| Author(s) | J.M. Páez Sandoval, D. I. Mijares Meza, A. S. De La Torre Gomez. |
| Institution | CETYS Universidad |
| Date | 29/05/2026 |

# 1. Introduction

## 1.1 Purpose

This document describes the design and architecture of the uart_driver module for Lab 05. It covers the bare-metal USART2 initialization sequence, character and string transmission, integer and float formatting helpers, and the mapping to the STM32F411RE ST-Link Virtual COM Port (VCP).

## 1.2 Scope

The uart_driver is a low-level bare-metal module for the STM32F411RE USART2 peripheral. It operates in transmit-only mode using polling (no interrupts, no DMA). It is fixed at 115200 baud with 8N1 framing. The module is used by main.c to stream real-time telemetry — attitude angles, PID outputs, and motor PWM values — over the ST-Link USB connection to a host serial terminal. GPIO configuration for PA2 (TX) is performed internally by uart_init().

## 1.3 Definitions, Acronyms, and Abbreviations

| Term | Definition |
| --- | --- |
| UART / USART | Universal (Synchronous/) Asynchronous Receiver/Transmitter |
| VCP | Virtual COM Port — USART2 is routed through the ST-Link USB bridge on the Nucleo board |
| PA2 | GPIO pin A2 — USART2_TX (AF7). Not physically accessible; routed to ST-Link internally |
| 8N1 | 8 data bits, No parity, 1 stop bit — standard serial framing |
| BRR | Baud Rate Register — USART2->BRR. Controls clock division for target baud rate |
| TXE | Transmit Data Register Empty flag (SR bit 7) — set when DR can accept next byte |
| TC | Transmission Complete flag (SR bit 6) — set when shift register is empty |
| AF7 | Alternate Function 7 — USART2 TX/RX function for PA2/PA3 on STM32F4 |
| TE | Transmitter Enable bit (CR1 bit 3) — must be set to transmit |
| UE | USART Enable bit (CR1 bit 13) — enables the USART peripheral |

## 1.4 Requirements

All requirements are allocated in Docs/SRS/UART_Driver_SRS.md (provided by instructor without modification).

# 2. System Architecture

## 2.1 High-Level Design

The uart_driver sits directly above the USART2 hardware registers. GPIO pin configuration for PA2 is performed internally by uart_init() using direct register access. The driver has no dependencies other than stm32f411xe.h.

| Layer | Modules |
| --- | --- |
| Application | main.c — calls uart_sendLine(), uart_sendFloat(), uart_sendInt() for telemetry |
| UART Driver | uart_init(), uart_sendChar(), uart_sendString(), uart_sendFloat(), uart_sendInt(), uart_sendLine() |
| Hardware | STM32F411RE USART2 (APB1). PA2 = TX (AF7), routed to ST-Link VCP. 115200 baud, 8N1. |

## 2.2 Module Interfaces

Public API declared in UART.h. Dependency: stm32f4xx.h only. No HAL. The driver configures PA2 as AF7 output internally — the caller does not need to configure GPIO before calling uart_init().

# 3. Detailed Design

## 3.1 Module Behavior

Figure 1. Sequence Diagram — uart_init(): clock enable (GPIOA + USART2), PA2 mode/AF/speed configuration, BRR write, CR1 TE+UE enable.

Figure 2. Activity Diagram — uart_sendChar(): poll TXE flag, write byte to DR.

Figure 3. Activity Diagram — uart_sendFloat(): extract sign, integer part, decimal digits iteratively.

Figure 4. Activity Diagram — uart_sendChar(): polls TXE flag, writes byte to DR.

Figure 5. Activity Diagram — uart_sendFloat(): extracts sign, integer part, and decimal digits iteratively.

Figure 3. Activity Diagram — uart_init(): clock enable, GPIO AF7 configuration, BRR write, TE+UE enable.

## 3.2 UART Driver Module Description

uart_init() enables the APB2 clock for GPIOA and the APB1 clock for USART2. It configures PA2 as Alternate Function mode (MODER=10), selects AF7 in AFRL, sets output speed to HIGH, and enables the internal pull-up. It then writes BRR = 0x008B (derived from 16 MHz HSI: mantissa=8, fraction=11) for 115200 baud, sets TE and UE in CR1, and returns. No interrupt or DMA bits are set.

uart_sendChar() polls the TXE flag in USART2->SR. Once set, it writes the byte to USART2->DR. The hardware clears TXE automatically.

uart_sendString() iterates over a null-terminated string calling uart_sendChar() for each byte.

uart_sendInt() handles the sign bit, then extracts decimal digits into a local buffer using modulo/divide by 10, reverses the buffer, and transmits each character. A zero input transmits '0' directly.

uart_sendFloat() calls uart_sendInt() for the integer part, transmits a decimal point, then iterates over the requested number of decimal digits by multiplying the fractional part by 10 and extracting each digit individually.

uart_sendLine() calls uart_sendString() followed by uart_sendChar('\r') and uart_sendChar('\n') to terminate the line for both Windows and Unix terminals.

# 4. Application Programming Interfaces

## 4.1 Data Types and Macros

| Macro / Constant | Value | Notes |
| --- | --- | --- |
| UART2_BRR | 0x008B | BRR for 115200 baud at 16 MHz HSI. Mantissa=8, Fraction=11 |
| (none) | — | No handle struct — module uses USART2 directly (single instance) |

## 4.2 Error Codes

The uart_driver does not define error codes. All functions are void or return void. Transmit errors (e.g., stuck TXE) result in an infinite polling loop; the caller cannot recover without a system reset. This is acceptable for a debug-only telemetry channel.

## 4.3 Function Definitions

### uart_init

| Function Name | uart_init |
| --- | --- |
| Syntax | void uart_init(void) |
| Parameters (in) | None |
| Return value | void |
| Description | Enables GPIOA and USART2 clocks. Configures PA2 as AF7 (USART2_TX) with HIGH speed and pull-up. Writes BRR = 0x008B for 115200 baud at 16 MHz. Sets CR1: TE=1, UE=1. |
| Constraints | Must be called before any other uart_* function. USART2 clock is on APB1 (RCC->APB1ENR bit 17). GPIOA clock is on AHB1 (RCC->AHB1ENR bit 0). |
| Available via | UART.h |

### uart_sendChar

| Function Name | uart_sendChar |
| --- | --- |
| Syntax | void uart_sendChar(char c) |
| Parameters (in) | c — byte to transmit |
| Return value | void |
| Description | Polls USART2->SR bit 7 (TXE) until set, then writes c to USART2->DR. Blocks until the data register is ready. |
| Constraints | Blocking — will spin indefinitely if USART2 is not initialized or the peripheral hangs. Call uart_init() first. |
| Available via | UART.h |

### uart_sendString

| Function Name | uart_sendString |
| --- | --- |
| Syntax | void uart_sendString(const char *s) |
| Parameters (in) | s — null-terminated C string to transmit |
| Return value | void |
| Description | Iterates over s calling uart_sendChar() for each byte until '\0' is reached. Does not transmit the null terminator. |
| Constraints | s must be null-terminated. Passing NULL is undefined behavior. |
| Available via | UART.h |

### uart_sendInt

| Function Name | uart_sendInt |
| --- | --- |
| Syntax | void uart_sendInt(int32_t val) |
| Parameters (in) | val — signed 32-bit integer to transmit as decimal ASCII |
| Return value | void |
| Description | Transmits the '-' character if val < 0, then negates val. Handles val == 0 by transmitting '0'. Otherwise extracts digits least-significant-first into a 12-byte buffer, reverses, and transmits. |
| Constraints | Maximum value: INT32_MIN (-2147483648) through INT32_MAX (2147483647). Output is always decimal (base 10). |
| Available via | UART.h |

### uart_sendFloat

| Function Name | uart_sendFloat |
| --- | --- |
| Syntax | void uart_sendFloat(float val, uint8_t decimals) |
| Parameters (in) | val — float value to transmit. decimals — number of decimal places (0–6 recommended) |
| Return value | void |
| Description | Transmits '-' if val < 0, then calls uart_sendInt() for the integer part, transmits '.', then iterates decimals times multiplying the fractional part by 10 and transmitting each digit. |
| Constraints | No rounding — truncates the last digit. Large decimals values (>6) may produce garbage due to float precision limits. NaN and Inf are not handled. |
| Available via | UART.h |

### uart_sendLine

| Function Name | uart_sendLine |
| --- | --- |
| Syntax | void uart_sendLine(const char *s) |
| Parameters (in) | s — null-terminated C string |
| Return value | void |
| Description | Calls uart_sendString(s) then transmits '\r' and '\n'. Produces CRLF line termination compatible with PuTTY, CoolTerm, and minicom. |
| Constraints | s must be null-terminated. Passing NULL is undefined behavior. |
| Available via | UART.h |

## 4.4 Internal Function Definitions

The uart_driver has no static helper functions. All USART2 protocol steps (clock enable, GPIO config, BRR write, poll-and-transmit) are implemented inline in the public API functions.

# 5. Future Enhancements

- Add DMA-based TX to free the CPU during string transmissions (currently each byte blocks ~8.7 μs at 115200 baud).

- Add interrupt-driven TX with a circular buffer to prevent telemetry from stalling the 1 kHz PID loop.

- Add uart_sendHex() for printing raw register values during hardware debugging.

- Add RX support (PA3, AF7) with a ring buffer for future RC receiver input over UART.

- Implement printf-style uart_printf() using a lightweight formatting layer to reduce call-site verbosity in main.c.

- Add configurable baud rate parameter to uart_init() to support 921600 baud for higher-throughput Blackbox logging.